NEW Ford PUMA Specifications

Performance and Economy

|  |  |  | CO2 g/km NEDC | Fuel consumption l/100 km NEDC |  |  | Fuel consumption l/100 km WLTP | CO2 g/km WLTP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Petrol engines | Power PS | Wheel size inch | Combined | Urban | Extra Urban | Combined | Overall | Overall |
| 1.0-litre EcoBoost 6-speed manual | 95 | 16 | 102 | 5.4 | 4.0 | 4.5 | 5.7 | 129 |
|  |  | 17 | 102 | 5.4 | 4.0 | 4.5 | 5.7 | 130 |
|  |  | 18 | 102 | 5.5 | 4.0 | 4.5 | 5.7 | 130 |
|  |  | 19 | 104 | 5.5 | 4.1 | 4.6 | 6.1 | 139 |
| 1.0-litre EcoBoost 6-speed manual | 125 | 16 | 103 | 5.4 | 4.0 | 4.5 | 5.7 | 131 |
|  |  | 17 | 103 | 5.4 | 4.0 | 4.5 | 5.8 | 132 |
|  |  | 18 | 103 | 5.4 | 4.0 | 4.5 | 5.8 | 131 |
|  |  | 19 | 106 | 5.5 | 4.1 | 4.6 | 6.2 | 141 |
| 1.0-litre EcoBoost Hybrid 6-speed manual | 125 | 16 | 96 | 4.9 | 3.9 | 4.2 | 5.4 | 124 |
|  |  | 17 | 96 | 4.9 | 3.9 | 4.2 | 5.5 | 124 |
|  |  | 18 | 96 | 4.9 | 3.9 | 4.2 | 5.4 | 124 |
|  |  | 19 | 99 | 5.0 | 4.0 | 4.3 | 5.8 | 132 |
| 1.0-litre EcoBoost Hybrid 6-speed manual | 155 | 16 | 99 | 5.1 | 3.9 | 4.4 | 5.5 | 126 |
|  |  | 17 | 99 | 5.1 | 3.9 | 4.4 | 5.6 | 127 |
|  |  | 18 | 99 | 5.1 | 3.9 | 4.4 | 5.6 | 127 |
|  |  | 19 | 101 | 5.1 | 4.1 | 4.5 | 5.9 | 133 |

|  |  | Performance |  |  |
| --- | --- | --- | --- | --- |
| Petrol engines | Power PS | Max speed km/h | 0-100 km/h sec | 50-100 km/h sec* |
| 1.0-litre EcoBoost 6-speed manual | 95 | 175 (109) | 11.9 | 13.3 |
| 1.0-litre EcoBoost 6-speed manual | 125 | 191 (119) | 10.0 | 9.7 |
| 1.0-litre EcoBoost Hybrid 6-speed manual | 125 | 191 (119) | 9.8 | 9.6 |
| 1.0-litre EcoBoost Hybrid 6-speed manual | 155 | 205 (127) | 9.0 | 8.4 |

* In 4th gear

WEIGHTS AND DIMENSIONS

|  | Kerb weight (kg)# | Gross Vehicle Mass (kg) | Gross Train Mass (kg) | Max. Towable Mass (braked) (kg) | Max. Towable Mass (unbraked) (kg) | Nose weight (kg) | Roof load (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0 EcoBoost 95 PS 6-speed manual | 1269 | 1765 | 2665 | 900 | 640 | 75 | 50 |
| 1.0 EcoBoost 125 PS 6-speed manual | 1280 | 1760 | 2860 | 1100 | 640 | 75 | 50 |
| 1.0 EcoBoost 125 PS EcoBoost Hybrid 6-speed manual | 1280 | 1760 | 2860 | 1100 | 640 | 75 | 50 |
| 1.0 EcoBoost 155 PS EcoBoost Hybrid 6-speed manual | 1280 | 1760 | 2860 | 1100 | 640 | 75 | 50 |

# Represents the lightest kerbweight assuming driver at 75 kg, full fluid levels and 90 per cent fuel levels, subject to manufacturing tolerances and options, etc., fitted. Weights represent base model specification without panoramic glass roof.

Towing limits quoted represent the maximum towing ability of the vehicle at its Gross Vehicle Mass to restart on a 12 per cent gradient at sea level. The performance and economy of all models will be reduced when used for towing.  Gross Train Mass includes trailer weight.

Dimensions

| Dimensions (mm unless stated) | Trend/ Titanium | Trend/ Titanium EcoBoost Hybrid | ST-Line | ST-Line EcoBoost Hybrid |
| --- | --- | --- | --- | --- |
| Exterior |  |  |  |  |
| Overall length without tow bar | 4186 | 4186 | 4207 | 4207 |
| Overall width mirrors extended/folded | 1930/1805 | 1930/1805 | 1930/1805 | 1930/1805 |
| Overall height (with shark fin antenna, unladen) | 1550 | 1554 | 1548 | 1552 |
| Wheelbase | 2588 | 2588 | 2588 | 2588 |
| Front track | 1567 | 1567 | 1562 | 1562 |
| Rear track | 1526 | 1526 | 1521 | 1521 |
| Front overhang | 850 | 850 | 856 | 856 |
| Rear overhang | 748 | 748 | 763 | 763 |
| Min ground clearance (GVM) | 140 | 143 | 139 | 142 |
| Min ground clearance (kerb) | 166 | 166 | 164 | 164 |
| Approach Angle (degrees) | 17.5 | 17.4 | 16.8 | 16.9 |
| Departure Angle (degrees) | 26.6 | 27.0 | 26.2 | 26.6 |
| Interior |  |  |  |  |
| Front headroom (without panorama roof) | 1000 | 1000 | 1000 | 1000 |
| Front headroom (with panorama roof) | 983 | 983 | 983 | 983 |
| Front legroom (maximum with seat in rear-most mid-height position) | 1127 | 1127 | 1127 | 1127 |
| Front shoulder room | 1348 | 1348 | 1348 | 1348 |
| Front hip room | 1311 | 1311 | 1311 | 1311 |
| Rear headroom (without panorama roof) | 965 | 965 | 965 | 965 |
| Rear legroom (maximum with seat in rear-most mid-height position) | 877 | 877 | 877 | 877 |
| Rear shoulder room | 1320 | 1320 | 1320 | 1320 |
| Rear hip room | 1278 | 1278 | 1278 | 1278 |
| Luggage capacity (litres) ‡ |  |  |  |  |
| 5-seat mode, laden to package tray (with tyre repair kit) | 456 | 401 | 456 | 401 |
| 2-seat mode, laden to roof (with tyre repair kit) | 1216 | 1161 | 1216 | 1161 |
| Luggage compartment dimensions |  |  |  |  |
| Maximum loading height | 865 | 810 | 865 | 810 |
| Load length at floor to 1st row (tyre repair kit) | 1472 | 1472 | 1472 | 1472 |
| Load length at floor to 2nd row seats | 725 | 725 | 725 | 725 |
| Load width between wheelhouses | 1000 | 1000 | 1000 | 1000 |
| Load opening width (floor) | 982 | 982 | 982 | 982 |
| Lift-over height (kerb) | 771 | 776 | 769 | 774 |
| Fuel tank capacity (litres) |  |  |  |  |
| Petrol | 42 | 42 | 42 | 42 |

‡ Measured in accordance with ISO 3832. Dimensions may vary dependent on the model and equipment fitted.

DRIVER ASSISTANCE TECHNOLOGIES≠

Active Park Assist with Perpendicular Park Assist

Adaptive Cruise Control with Stop & Go, Speed Sign Recognition and Lane-Centring (available with 7-speed automatic transmission only)

Adjustable Speed Limiter

Auto high beam

Auto lighting and rain-sensing wipers

Blind Spot Information System

Cross Traffic Alert with Active Braking

Distance Indication

Driver Alert

Evasive Steering Assist

Forward Collision Warning

Front and rear parking sensors

Hill Launch Assist

Intelligent Speed Limiter

Local Hazard Information

Lane Keeping Aid with Road Edge Detection

Lane Keeping Alert

Post-Collision Braking

Pre-Collision Assist with Pedestrian and Cyclist Detection

Rear wide-view camera

Selectable Drive Modes

Traffic Sign Recognition

Wrong Way Alert

COMFORT AND CONVENIENCE≠

B&O Sound System

Ford MegaBox

FordPass Connect on-board modem

Hands-free tailgate

Lumbar massage seats

Openable panoramic roof

Removable seat covers

SYNC 3 with 8-inch touchscreen

Wireless charging pad

≠Feature availability dependent on vehicle specification

STEERING

| System | Rack and pinion with Electronic Power Assisted Steering (EPAS) |
| --- | --- |
| Ratio | 15.1:1 |
| Turning circle (m) kerb-to-kerb | 10.5 |

CHASSIS

| Front suspension | Independent. MacPherson struts, L-shaped lower control arm, steering gear and hollow stabiliser bar mounted on subframe |
| --- | --- |
| Rear suspension | Twistbeam rear suspension with toe-correcting angled bushes, springs located below floor and separate twintube shock absorbers |

BRAKES

|  | Front | Rear |
| --- | --- | --- |
| Braking | Hydraulically operated dual-circuit system with diagonal distribution. Vented front discs. Solid rear discs or drums. Electronic four-channel anti-lock braking system (ABS) with electronic brake-force distribution (EBD), Electronic Stability System (ESP) and Emergency Brake Assist (EBA). Optional autonomous emergency braking (AEB) as part of Pre-Collision Assist with Pedestrian and Cyclist Detection and Cross Traffic Alert with Active Braking |  |
| Disc/Drum dimensions (mm) | Ø278 x 25 | Ø271 x 11 disc Ø 228 x 40 drum |
| Piston dimensions (mm) | Ø54 | Ø36 |

WHEELS & TYRES

6.0 x 16-inch with 205/65 R16

6.5 x 16-inch with 205/65 R16

7 x 17-inch with 215/55 R17

7 x 18-inch with 215/50 R18

7.5 x 19-inch with 225/40 R19

PETROL ENGINES

|  |  | 1.0-litre EcoBoost (95, 125 PS) |  |
| --- | --- | --- | --- |
| Type |  | In-line three cylinder turbo petrol, Ti-VCT, transverse |  |
| Displacement | cm3 | 999 |  |
| Bore | Mm | 71.9 |  |
| Stroke | Mm | 82.0 |  |
| Compression ratio |  | 11.5:1 | 10.5:1 |
| Max power | PS (kW) | 95 (70) | 125 (92) |
|  | at rpm | 4000-6000 | 6000 |
| Max torque steady state | Nm | 170 | 170 |
|  | at rpm | 1750-3900 | 1400-4500 |
| Max torque overboost | Nm | N/A | 200 |
|  | at rpm | N/A | 1750 |
| Valve gear |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |  |
| Cylinders |  | 3 in-line, cylinder deactivation technology |  |
| Cylinder head |  | Cast aluminium |  |
| Cylinder block |  | Cast iron |  |
| Camshaft drive |  | Chain drive with hydraulic tensioner |  |
| Crankshaft |  | Cast iron, 6 counterweights, 4 main bearings |  |
| Engine management |  | Bosch MED17 with CAN-Bus and individual cylinder knock control. FGEC software |  |
| Fuel injection |  | High pressure direct fuel injection with 5 hole injectors |  |
| Emission level |  | Euro 6d-TEMP |  |
| Emission control |  | Rapid light-off catalyst, gasoline particulate filter |  |
| Turbocharger |  | Fixed geometry turbocharger |  |
| Lubrication system |  | Electronically controlled variable displacement oil pump for improved fuel economy |  |
| Cooling system |  | Split cooling system with 2 thermostats. Cam driven water pump for reduced power consumption |  |
| Transmission |  | 6-speed manual |  |
| Gear ratios |  | 6th 0.63 5th 0.76 4th 0.94 3rd 1.28 2nd 1.96 1st 3.42 R 3.83 FDR 4.35 |  |

|  |  | 1.0-litre EcoBoost Hybrid (125, 155 PS) |  |
| --- | --- | --- | --- |
| Type |  | In-line three cylinder turbo petrol, Ti-VCT, transverse |  |
| Displacement | cm3 | 999 |  |
| Bore | Mm | 71.9 |  |
| Stroke | Mm | 82.0 |  |
| Compression ratio |  | 10.5:1 | 10.0:1 |
| Max power | PS (kW) | 125 (92) | 155 (114) |
|  | at rpm | 6000 | 6000 |
| Max torque steady state | Nm | 170 | 190 |
|  | at rpm | 1400-4500 | 1900-5500 |
| Max torque overboost | Nm | 200 | 220 |
|  | at rpm | 1750 | 3000 |
| Max torque e-assist sport mode | Nm | 210 | 240 |
|  | at rpm | 1750 | 2500 |
| Valve gear |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |  |
| Cylinders |  | 3 in-line, cylinder deactivation technology |  |
| Cylinder head |  | Cast aluminium |  |
| Cylinder block |  | Cast iron |  |
| Camshaft drive |  | Chain drive with hydraulic tensioner |  |
| Crankshaft |  | Cast iron, 6 counterweights, 4 main bearings |  |
| Engine management |  | Bosch MED17 with CAN-Bus and individual cylinder knock control. FGEC software |  |
| Fuel injection |  | High pressure direct fuel injection with 5 hole injectors |  |
| Emission level |  | Euro 6d-TEMP |  |
| Emission control |  | Rapid light-off catalyst, gasoline particulate filter |  |
| Turbocharger |  | Fixed geometry turbocharger |  |
| Electric motor |  | 11.5kW Belt Integrated Starter-Generator |  |
| Battery |  | 48V 10Ah Li-ion air cooled battery pack |  |
| Lubrication system |  | Electronically controlled variable displacement oil pump for improved fuel economy |  |
| Cooling system |  | Split cooling system with 2 thermostats. Cam driven water pump for reduced power consumption |  |
| Transmission |  | 6-speed manual |  |
| Gear ratios |  | 6th 0.63 5th 0.76 4th 0.94 3rd 1.28 2nd 1.96 1st 3.42 R 3.83 FDR 4.35 | 6th 0.63 5th 0.76 4th 0.94 3rd 1.28 2nd 1.96 1st 3.42 R 3.83 FDR 4.58 |

# # #

Note: The data information in this press release reflects preliminary specifications and was correct at the time of going to print. However, Ford policy is one of continuous product improvement. The right is reserved to change these details at any time.

The declared fuel/energy consumptions, CO2 emissions and electric range are measured according to the technical requirements and specifications of the European Regulations (EC) 715/2007 and (EC) 692/2008 as last amended. Fuel consumption and CO2 emissions are specified for a vehicle variant and not for a single car. The applied standard test procedure enables comparison between different vehicle types and different manufacturers. In addition to the fuel efficiency of a car, driving behaviour as well as other non-technical factors play a role in determining a car's fuel/energy consumption, CO2 emissions and electric range. CO2 is the main greenhouse gas responsible for global warming.

Since 1 September 2017, certain new vehicles are being type-approved using the World Harmonised Light Vehicle Test Procedure (WLTP) according to (EU) 2017/1151 as last amended, which is a new, more realistic test procedure for measuring fuel consumption and CO2 emissions. Since 1 September 2018 the WLTP has begun replacing the New European Drive Cycle (NEDC), which is the outgoing test procedure. During NEDC Phase-out, WLTP fuel consumption and CO2 emissions are being correlated back to NEDC. There will be some variance to the previous fuel economy and emissions as some elements of the tests have altered i.e., the same car might have different fuel consumption and CO2 emissions.

About Ford Motor Company

Ford Motor Company is a global company based in Dearborn, Michigan. The company designs, manufactures, markets and services a full line of Ford cars, trucks, SUVs, electrified vehicles and Lincoln luxury vehicles, provides financial services through Ford Motor Credit Company and is pursuing leadership positions in electrification, autonomous vehicles and mobility solutions. Ford employs approximately 194,000 people worldwide. For more information regarding Ford, its products and Ford Motor Credit Company, please visit www.corporate.ford.com.

Ford of Europe is responsible for producing, selling and servicing Ford brand vehicles in 50 individual markets and employs approximately 50,000 employees at its wholly owned facilities and consolidated joint ventures and approximately 64,000 people when unconsolidated businesses are included. In addition to Ford Motor Credit Company, Ford Europe operations include Ford Customer Service Division and 20 manufacturing facilities (13 wholly owned facilities and seven unconsolidated joint venture facilities). The first Ford cars were shipped to Europe in 1903 – the same year Ford Motor Company was founded. European production started in 1911.

|  | Contact:; Dan Jones. Ford of Europe. +44 (0) 1268 401 917. djone602@ford.com.. |
| --- | --- |