2024 Ford KUGA Specifications

Performance and Economy

| Kuga |  | Fuel consumption l/100 km WLTP 1 |  | Performance |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Engine | Power PS | CO2 (g/km) 1 | Combined | Max Speed (km/h) | 0-100 km/h 0-62 mph (sec) | 50-100 km/h 31-62 mph (sec) 2 |
| 1.5-litre EcoBoost FWD (6-speed manual) | 150 | 144-163 | 6.3-7.2 | 195 | 9.5 | 9.5 |
| 2.5-litre Petrol Hybrid FWD | 180 | 121-143 | 5.3-6.2 | 196 | 9.1 | N/A |
| 2.5-litre Petrol Hybrid AWD | 183 | 130-146 | 5.7-6.4 | 196 | 8.3 | N/A |
| 2.5-litre Petrol Plug-In Hybrid FWD | 243 | 20-28 | 0.9-1.2 | 200 | 7.3 | N/A |
|  |  | Equivalent all-electric range overall WLTP | 60-69 km |
|  |  | Equivalent all-electric range city WLTP | 88 km |

1 CO2 emission and fuel efficiency ranges may vary according to vehicle variants offered by individual markets.

The declared WLTP fuel/energy consumptions, CO2-emissions and electric range are determined according to the technical requirements and specifications of the European Regulations (EC) 715/2007 and (EU) 2017/1151 as last amended. The applied standard test procedures enable comparison between different vehicle types and different manufacturers.

2 In 4th gear

WEIGHTS

|  | Kerb weight (kg) 3 | Gross Vehicle Mass (kg) | Gross Train Mass (kg) | Max. Towable Mass (braked) (kg) 4 | Max. Towable Mass (unbraked) (kg) 4 | Max. Roof Load (kg) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.5-litre EcoBoost FWD (150 PS) | 1526 | 2075 | 3805 | 1730 | 750 | 75 |
| 2.5-litre Petrol Hybrid FWD (180 PS) | 1689 | 2230 | 3830 | 1600 | 750 | 75 |
| 2.5-litre Petrol Hybrid AWD (183 PS) | 1765 | 2315 | 4415 | 2100 | 750 | 75 |
| 2.5-litre Petrol Plug-In Hybrid FWD (243 PS) | 1859 | 2400 | 4500 | 2100 | 750 | 75 |

3 Represents the lightest kerbweight assuming driver at 75 kg, full fluid levels and 90 per cent fuel levels, subject to manufacturing tolerances and options, etc., fitted. Weights represent base model specification without panoramic glass roof.

4 Towing limits quoted represent the maximum towing ability of the vehicle at its Gross Vehicle Mass to restart on a 12 per cent gradient at sea level. The performance and economy of all models will be reduced when used for towing.  Gross Train Mass includes trailer weight. Max towing varies based on cargo, vehicle configuration, accessories and number of passengers.

DIMENSIONS 5

|  |  | Kuga Titanium | Kuga ST-Line | Kuga Active |
| --- | --- | --- | --- | --- |
| ##### Overall length | mm | 4604 | 4615 | 4645 |
| Overall width with mirrors | mm | 2177 | 2177 | 2177 |
| Overall width without mirrors | mm | 1882 | 1882 | 1882 |
| Overall width with mirrors folded | mm | 2000 | 2000 | 2000 |
| Overall height (maximum) | mm | 1679 | 1651 | 1683 |
| Wheelbase | mm | 2711 | 2711 | 2711 |
| ##### Track front | mm | 1599 | 1599 | 1599 |
| ##### Track rear | mm | 1590 | 1590 | 1590 |
| ##### Approach angle | deg | 20.0 | 18.4 | 20.0 |
| ##### Departure angle | deg | 24.7 | 23.5 | 24.7 |
| ##### Minimum running ground clearance - kerb | mm | 166-174 | 152-176 | 168-191 |
| ##### Ground clearance at maximum load | mm | 143-160 | 129-159 | 144-171 |
| ##### Interior First Row |  |  |  |  |
| ##### Headroom without sunroof | mm | 1021 | 1021 | 1021 |
| ##### Headroom with sunroof | mm | 985 | 985 | 985 |
| Legroom | mm | 1076 | 1076 | 1076 |
| Shoulder room | mm | 1464 | 1464 | 1464 |
| Hip room | mm | 1405 | 1405 | 1405 |
| Interior Second Row |  |  |  |  |
| Headroom without sunroof | mm | 998 | 998 | 998 |
| ##### Headroom with sunroof | mm | 980 | 980 | 980 |
| ##### Nominal leg room | mm | 986 | 986 | 986 |
| ##### Maximum leg room | mm | 1035 | 1035 | 1035 |
| ##### Shoulder room | mm | 1423 | 1423 | 1423 |
| ##### Hip room | mm | 1352 | 1352 | 1352 |
| ##### Cargo Compartment |  |  |  |  |
| ##### Lift in height | mm | 713-722 | 704-717 | 714-726 |
| ##### Cargo height | mm | 885 | 885 | 885 |
| ##### Rear opening height | mm | 773 | 773 | 773 |
| ##### Loading length at floor to first row | mm | 1688 | 1688 | 1688 |
| ##### Load width between wheel arches | mm | 1051 | 1051 | 1051 |
| ##### Load opening width at floor | mm | 1071 | 1071 | 1071 |
| ##### Luggage capacity 6 |  |  |  |  |
| ##### Cargo volume behind first row (2-seat mode; laden to roof) with tyre repair kit 7 | litres | 1534 | 1534 | 1534 |
| ##### Cargo length at floor behind second row with tyre repair kit | mm | 865-1017 | 865-1017 | 865-1017 |
| ##### Closed cargo compartment second row (under cargo shade) with tyre repair kit (min-max depending on sliding second row position) | litres | 412-553 | 412-553 | 412-553 |

5 Front-wheel drive with 17- to 20-inch wheels

6 Cargo and load capacity limited by weight and weight distribution.

7 V214-1. Dimensions may vary dependent on the model and equipment fitted.

DRIVER ASSISTANCE TECHNOLOGIES 8, 9

Active Park Assist 2

Adaptive Cruise Control with Stop & Go, Speed Sign Recognition, Lane Centring and Predictive Speed Assist

Auto High Beam Control

Auto lighting

Blind Spot Information System (BLIS) with Trailer Tow

Cross Traffic Alert with Active Braking

Distance Indication

Driver Alert

Dynamic Pixel LED headlights with Adaptive Front Lighting

Emergency Brake Assist

Evasive Steering Assist

Forward Collision Warning with Brake Support

Front and Rear Park Aid

Front wide-view camera

Glare-Free High Beam

Hill Start Assist

Intelligent Speed Assist

Intersection Assist

Lane Keeping Aid with Lane Departure Warning

Local Hazard Information

Post-Collision Braking

Pre-Collision Assist with Active Braking

Rain-sensing wipers

Rear wide-view camera

Rear Occupant Alert

Reverse Brake Assist

Selectable Drive Modes

Surround View Camera System

Traffic Sign Recognition

Trailer Tow Navigation

Wrong Way Alert

COMFORT AND CONVENIENCE 8, 9

12.3-inch LCD instrument cluster

Wireless Apple CarPlay and Android Auto

B&O Sound System

FordPass 5G modem

Hands-free tailgate

Head-up display

Openable panoramic roof

Retractable tow bar

SYNC 4 with 13.2-inch touchscreen

Wireless charging pad

8 Feature availability dependent on vehicle specification

9 Driver-assist features are supplemental to and do not replace the driver’s attention, judgement and need to control the vehicle

STEERING

| System | Column-mounted electric power-assisted steering, mechanical steering gear |
| --- | --- |
| Ratio | 13.75:1 |
| Turning circle (m) kerb-to-kerb | 11.38 |

CHASSIS

| Front suspension | Independent MacPherson struts, L-shaped lower control arm, steering gear and hollow stabilizer bar mounted on subframe |
| --- | --- |
| Rear suspension | Isolated subframe for the Short Long Arm (SLA) independent (AWD) twist-beam rear axle (FWD) suspension, multi-compound bushes with different stiffness characteristics separating the subframe from the body structure, springs located below floor, hollow stabilizer |

BRAKES

|  | Front | Rear |
| --- | --- | --- |
| Braking | Hydraulically operated dual-circuit system with diagonal distribution. Vented front discs. Solid rear discs with Electric Park Brake system (EPB). Electronic four-channel anti-lock braking system (ABS) with electronic brake-force distribution (EBD). Electronic brake booster on hybrid powertrains and for selected driver assistance technologies. Electronic stability control system (ESC), including roll stability control (RSC) and trailer sway control (TSC). |  |
| Disc dimensions (mm) | Ø308 x 27 Ø330 x 27 | Ø302 x 11 |
| Piston dimensions (mm) | Ø57 2 x Ø44 | Ø36 (iEPB) |

WHEELS & TYRES

7 x 17-inch with 225/65 R17

7.5 x 18-inch with 225/60 R18

7.5 x 19-inch with 225/55 R19

8.0 x 20-inch with 245/45 R20

PETROL ENGINES

|  |  | 1.5-litre EcoBoost (150 PS) |
| --- | --- | --- |
| Type |  | In-line three-cylinder turbo petrol, Ti-VCT, transverse |
| Displacement | cm3 | 1497 |
| Bore | mm | 84.0 |
| Stroke | mm | 90.0 |
| Compression ratio |  | 11.0:1 |
| Max power | PS (kW) | 150 (110) |
|  | at rpm | 6000 |
| Max torque steady state | Nm | 240 |
|  | at rpm | 1600-4000 |
| Valve gear |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |
| Cylinders |  | 3 in-line |
| Cylinder head |  | Cast aluminium with iron liners |
| Cylinder block |  | Cast aluminium |
| Camshaft drive |  | Chain driven with hydraulic tensioner |
| Crankshaft bearings |  | Cast iron, 6 counterweights, 4 main bearings |
| Engine management |  | Bosch MG1CS016 with CAN-Bus and individual cylinder knock control. FGEC software |
| Fuel injection |  | 3 high-pressure direct fuel injection, 6-hole injectors (DI) plus 3 low-pressure port injectors (PFI) |
| Emission level |  | Euro 6d |
| Emission control |  | Rapid light-off catalyst, gasoline particulate filter |
| Turbocharger |  | Continental radial-axial (RAAX) turbocharger |
| Lubrication system |  | Two-stage variable displacement oil pump |
| Cooling system |  | Single thermostat |
| Transmission |  | 6-speed manual |
| Gear ratios |  | 6th 0.651 5th 0.775 4th 0.971 3rd 1.290 2nd 1.952 1st 3.583 R 3.333 FDR 4.563 |

HYBRID ENGINES

|  |  | 2.5-litre Petrol Hybrid (180/183 PS) |
| --- | --- | --- |
| Type |  | Atkinson Cycle port fuel injection engine |
| Motor |  | Permanent magnet 3-phase AC synchronous traction motor, liquid cooled |
| Max power | PS (kW) | 180 (FWD) 183 (AWD) (132/135) |
|  | at rpm | 5500 |
| Max torque | Nm | N/A |
|  | at rpm | N/A |
| Battery |  | Lithium-ion |
| High-voltage cooling system |  | Liquid-cooled |
| Battery capacity | kWh | 1.1 kWh |
| Transmission |  | Ford-developed power-split automatic with continuously variable ratio |
| Gear ratio |  | AWD 3.368 FDR FWD 2.91 FDR |
| Fuel tank | litres | 54 |
| Displacement | cm3 | 2488 |
| Bore | mm | 89.0 |
| Stroke | mm | 100.0 |
| Compression ratio |  | 13.0:1 |
| Valve gear |  | DOHC with 4 valves per cylinder, variable cam timing |
| Cylinders |  | 4 inline |
| Cylinder head |  | Aluminium |
| Cylinder block |  | Aluminium |
| Camshaft drive |  | Chain-in-oil with dynamic tensioner |
| Crankshaft |  | Forged steel, 8 counter-weights, 5 main bearings |
| Engine management |  | Bosch MED17 with CAN-Bus and individual cylinder knock control. Bosch software |
| Fuel injection |  | Multi-port fuel injection |
| Emission level |  | Euro 6d |
| Lubrication system |  | Electronically controlled variable displacement oil pump |
| Engine cooling system |  | Liquid cooled, separate from high-voltage cooling/heating system |

|  |  | 2.5-litre Petrol Plug-In Hybrid (243 PS) |
| --- | --- | --- |
| Type |  | Atkinson Cycle port fuel injection engine |
| Motor |  | Permanent magnet 3-phase AC synchronous traction motor, liquid cooled |
| Max power | PS (kW) | 243 (179) |
|  | at rpm | N/A |
| Max torque | Nm | N/A |
|  | at rpm | N/A |
| Battery |  | Lithium-ion |
| High-voltage cooling system |  | Liquid-cooled/heated |
| Battery capacity | kWh | 14.4 kWh 48 Ah |
| Transmission |  | Ford-developed power-split automatic with continuously variable ratio |
| Gear ratio |  | 3.368 FDR |
| Fuel tank | litres | 42.7 |
| Displacement | cm3 | 2488 |
| Bore | mm | 89.0 |
| Stroke | mm | 100.0 |
| Compression ratio |  | 13.0:1 |
| Valve gear |  | DOHC with 4 valves per cylinder, variable cam timing |
| Cylinders |  | 4 inline |
| Cylinder head |  | Aluminium |
| Cylinder block |  | Aluminium |
| Camshaft drive |  | Chain-in-oil with dynamic tensioner |
| Crankshaft |  | Forged steel, 8 counter-weights, 5 main bearings |
| Engine management |  | Bosch MED17 with CAN-Bus and individual cylinder knock control. Bosch software |
| Fuel injection |  | Multi-port fuel injection |
| Emission level |  | Euro 6d |
| Lubrication system |  | Electronically controlled variable displacement oil pump |
| Engine cooling system |  | Liquid cooled, separate from high-voltage cooling/heating system |

### DRIVE LINE

| Drive options: | AWD: Electronically controlled Ford Intelligent All Wheel Drive with variable torque distribution |  |  |  |
| --- | --- | --- | --- | --- |
|  | FWD: Front-wheel drive |  |  |  |

# # #

Ford, a global American brand woven into the fabric of Europe for more than 100 years, is committed to freedom of movement that goes hand-in-hand with looking after the planet and each other. The company’s Ford+ plan, with Model e, Ford Pro and the Ford Blue business units is accelerating its European transformation to an all-electric and carbon neutral future by 2035. The company is driving forward with bold, new EVs, each one designed with European drivers in mind and innovating with services to help people connect, communities grow, and businesses thrive. Selling and servicing Ford vehicles in 50 individual European markets, operations also include the Ford Motor Credit Company, Ford Customer Service Division and 14 manufacturing facilities (eight wholly owned and six unconsolidated joint venture facilities) with four centres based in Cologne, Germany; Valencia, Spain and at our joint venture in Craiova, Romania and Kocaeli, Türkiye.  Ford employs approximately 34,000 people at its wholly owned facilities and consolidated joint ventures and approximately 57,000 people including unconsolidated businesses across Europe. More information about the company, its products and Ford Credit is available at corporate.ford.com.