2024 Ford PUMA and PUMA ST POWERSHIFT Specifications

Performance and Economy 1

| Puma |  |  |  | Performance |  |
| --- | --- | --- | --- | --- | --- |
| Engine | Power (PS) | CO2 (g/km) 2 | Fuel consumption l/100 km 2 | Max speed (km/h) | 0-100 km/h (sec) |
| 1.0-litre EcoBoost Hybrid (6-spd manual) | 125 | 119-135 | 5.3-6.0 | 191 | 9.8 |
| 1.0-litre EcoBoost Hybrid (7-spd Powershift automatic) | 125 | 5.6-6.3 | 127-143 | 190 | 9.6 |
| 1.0-litre EcoBoost Hybrid (7-spd Powershift automatic) | 155 | 5.5-6.3 | 125-142 | 200 | 8.7 |
| Puma ST Powershift |  |  |  | Performance |  |
| Engine | Power (PS) | CO2 (g/km) 2 | Fuel consumption l/100 km 2 | Max speed (km/h) | 0-100 km/h (sec) |
| 1.0-litre EcoBoost Hybrid (7-spd Powershift automatic) | 170 3 | 135-146 | 6.0-6.5 | 210 | 7.4 |

WEIGHTS AND DIMENSIONS 1

|  | Kerb weight (kg) 4 | Gross Vehicle Mass (kg) | Gross Train Mass (kg) | Max. Towable Mass (braked) (kg) 5 | Max. Towable Mass (unbraked) (kg) 5 | Nose weight (kg) | Roof load (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Puma |  |  |  |  |  |  |  |
| 125 PS 1.0-litre EcoBoost Hybrid (6-spd manual) | 1316 | 1785 | 2885 | 1100 | 640 | 75 | 50 |
| 125 PS 1.0-litre EcoBoost Hybrid (7-spd Powershift automatic) | 1368 | 1835 | 2735 | 900 | 665 | 75 | 50 |
| 155 PS 1.0-litre EcoBoost Hybrid (7-spd Powershift automatic) | 1368 | 1835 | 2735 | 900 | 665 | 75 | 50 |
| Puma ST Powershift |  |  |  |  |  |  |  |
| 170 PS 1.0-litre EcoBoost Hybrid 7-spd Powershift automatic | 1390 | 1850 | 2750 | 900 | 665 | 75 | 25 |

Dimensions 6

| Dimensions (mm unless stated) | Puma | Puma ST Powershift |
| --- | --- | --- |
| Exterior |  |  |
| Overall length without tow bar | 4186 | 4226 |
| Overall width mirrors extended/folded | 1930/1805 | 1930/1805 |
| Overall height (with shark fin antenna, unladen) | 1550 | 1533 |
| Wheelbase | 2588 | 2588 |
| Front track | 1567 | 1567 |
| Rear track | 1526 | 1523 |
| Front overhang | 850 | 875 |
| Rear overhang | 748 | 763 |
| Min ground clearance (kerb) | 163 | 152 |
| Approach Angle (degrees) | 17.5 | 13.8 |
| Departure Angle (degrees) | 26.6 | 24.0 |
| Interior |  |  |
| Front headroom (without panorama roof) | 1000 | 1000 |
| Front headroom (with panorama roof) | 983 | 983 |
| Front legroom (maximum with seat in rear-most mid-height position) | 1113 | 1113 |
| Front shoulder room | 1348 | 1348 |
| Front hip room | 1301 | 1301 |
| Rear headroom (without panorama roof) | 965 | 965 |
| Rear legroom (maximum with seat in rear-most mid-height position) | 863 | 863 |
| Rear shoulder room | 1320 | 1320 |
| Rear hip room | 1234 | 1234 |
| Luggage capacity (litres) 7 |  |  |
| 5-seat mode, laden to package tray (with tyre repair kit) | 456 | 456 |
| 2-seat mode, laden to roof (with tyre repair kit) | 1216 | 1216 |
| Luggage compartment dimensions |  |  |
| Maximum loading height | 865 | 865 |
| Load length at floor to 1st row (tyre repair kit) | 1472 | 1472 |
| Load length at floor to 2nd row seats | 725 | 725 |
| Load width between wheelhouses | 1000 | 1000 |
| Load opening width (floor) | 982 | 982 |
| Lift-over height (kerb) | 771 | 754 |
| Fuel tank capacity (litres) |  |  |
| Petrol | 42 | 42 |

ADVANCED DRIVER-ASSISTANCE SYSTEMS 8,9

360-degree surround-view camera system 10

Adjustable Speed Limiter

Auto high beam

Auto lighting and rain-sensing wipers

Cross Traffic Alert with Active Braking

Cruise Control

Driver Alert

Dynamic Matrix LED headlights

Enhanced Blind Spot Information System

Forward Collision Warning

Front and rear parking sensors

Glare-free High Beam

Hill Launch Assist

Intelligent Adaptive Cruise Control with Stop & Go and Lane Centring 11

Intelligent Speed Limiter

Intersection Assist 12

Local Hazard Information

Lane Keeping Alert

Post-Collision Braking

Pre-Collision Assist with Active Braking

Predictive Dynamic Bending Light

Predictive Speed Assist

Rear wide-view camera

Rear Cross Traffic Braking

Reverse Brake Assist

Selectable Drive Modes

Traffic Sign Recognition

Wrong Way Alert

COMFORT AND CONVENIENCE 9

12.8-inch digital instrument cluster

5G FordPass Connect modem 13

Adjustable lumbar support seats

Apple CarPlay and Android Auto (wireless integration) 14

Alexa Built-in 15

B&O audio system

Ford MegaBox

Hands-free tailgate

Heated front seats

Heated steering wheel

Openable panoramic roof

SYNC 4 with 12-inch touchscreen 16

Wireless charging pad 17

STEERING

|  | Puma | Puma ST Powershift |
| --- | --- | --- |
| System | Rack and pinion with Electronic Power Assisted Steering (EPAS) | Rack and pinion with Electronic Power Assisted Steering (EPAS) |
| Ratio | 15.1:1 | 11.4:1 |
| Turning circle (m) kerb-to-kerb | 10.4 | 11.2 |

CHASSIS

|  | Front suspension | Rear suspension |
| --- | --- | --- |
| Puma | Independent. MacPherson struts, L-shaped lower control arm, steering gear and hollow stabiliser bar mounted on subframe. | Twistbeam rear suspension with toe-correcting angled bushes, springs located below floor and separate twintube shock absorbers. |
| Puma ST Powershift | Independent suspension with MacPherson struts, Hitachi twin-tube frequency reactive shock absorbers and 24.2 mm x 6 mm diameter anti-roll bar. | Twist-beam rear suspension with toe-correcting angled bushes and 28 mm anti-roll bar. In-car stiffness of 2000 Nm/deg. Force vectoring springs located below floor and separate Hitachi twin-tube frequency reactive shock absorbers. |
| Puma ST Powershift Handling Pack | Independent suspension with MacPherson struts, KW twin-tube shock absorbers with dedicated setting for improved handling characteristics. Spring stiffness increased 20 per cent, height decreased 10 mm. Finished in Ford Performance Blue. 24.2 mm x 6 mm diameter anti-roll bar. | KW twin-tube shock absorbers with dedicated setting for improved handling characteristics. Spring stiffness increased 20 per cent, height decreased 10 mm. Finished in Ford Performance Blue. |

BRAKES

|  | Front | Rear |
| --- | --- | --- |
| Braking | Hydraulically operated dual-circuit system with diagonal distribution. Vented front discs. Solid rear discs or drums. Electronic four-channel anti-lock braking system (ABS) with electronic brake-force distribution (EBD), Electronic Stability System (ESP) and Emergency Brake Assist (EBA). Optional autonomous emergency braking (AEB). |  |
| Puma |  |  |
| Disc/drum dimensions (mm) | Ø278 x 25 | Ø271 x 11 disc Ø 228 x 40 drum |
| Piston dimensions (mm) | Ø54 | Ø36 |
| Puma ST Powershift |  |  |
| Disc dimensions (mm) | Ø325 x 27 | Ø271 x 11 |
| Piston dimensions (mm) | Ø57 | Ø36 |

WHEELS & TYRES

| Puma | Puma ST Powershift |
| --- | --- |
| 7 x 17-inch with 215/55 R17 7 x 18-inch with 215/50 R18 7.5 x 19-inch with 225/40 R19 | 8.0 x 19-inch alloy wheels with 225/40 R19 |

HYBRID ENGINES 1

|  |  | 1.0-litre EcoBoost Hybrid (125, 155, 170 PS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type |  | In-line three-cylinder turbo petrol, 48-volt mild hybrid, Ti-VCT, transverse |  |  |  |
| Displacement | cm3 | 999 |  |  |  |
| Bore | mm | 71.9 |  |  |  |
| Stroke | mm | 82.0 |  |  |  |
| Compression ratio |  | 10.5:1 |  | 10.0:1 | 10.0:1 |
| Max power | PS (kW) | 125 (92) |  | 155 (114) | 170 (125) 3 |
|  | at rpm | 6000 |  | 6000 | 5750 |
| Max torque steady state (ICE) | Nm | 170 |  | 190 | 200 |
|  | at rpm | 1400-4500 |  | 2100-5500 | 2000-5500 |
| Max torque performance | Nm | 210 |  | 240 | 248 3 |
|  | at rpm | 1750 |  | 2500 | 3000 |
| Valve gear |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |  |  |  |
| Cylinders |  | 3 in-line |  |  |  |
| Cylinder head |  | Cast aluminium |  |  |  |
| Cylinder block |  | Cast iron |  |  |  |
| Camshaft drive |  | Chain drive with hydraulic tensioner |  |  |  |
| Crankshaft |  | Cast iron, 6 counterweights, 4 main bearings |  |  |  |
| Engine management |  | Vitesco EMS2916 with CAN-BUS, individual cylinder knock control, FGEC software supporting FNV2.1 architecture |  |  |  |
| Fuel injection |  | 250 bar (max) high pressure direct fuel injection with 5-hole injectors |  |  |  |
| Emission level |  | Euro 6d |  |  |  |
| Emission control |  | Rapid light-off catalyst, gasoline particulate filter |  |  |  |
| Turbocharger |  | Fixed geometry turbocharger |  |  |  |
| Electric motor |  | 11.5kW Belt Integrated Starter-Generator |  |  |  |
| Battery |  | 48V 8Ah Li-ion air cooled battery pack |  |  |  |
| Lubrication system |  | Electronically controlled variable displacement oil pump for improved fuel economy |  |  |  |
| Cooling system |  | Split cooling system with 2 thermostats. Cam driven water pump for reduced power consumption |  |  |  |
| Transmission |  | 6-speed manual | 7-speed Powershift dual-clutch automatic |  |  |
| Gear ratios |  | 6th 0.634 5th 0.757 4th 0.943 3rd 1.276 2nd 1.958 1st 3.417 R 3.833 FDR 4.353 | 7th 0.638 6th 0.771 5th 0.851 4th 1.114 3rd 1.594 2nd 2.824 1st 4.462 R 2.824 FDR 4.176 (ratios 1,2,6,7) 4.733 (ratios 3,4,5,R) |  |  |

# # #

1 Power, torque, payload, towing and WLTP fuel economy ratings are independent attributes and may not be achieved simultaneously.

2 CO2 emission and fuel efficiency ranges may vary according to vehicle variants offered by individual markets.

The declared WLTP fuel/energy consumptions, CO2-emissions and electric range are determined according to the technical requirements and specifications of the European Regulations (EC) 715/2007 and (EU) 2017/1151 as last amended. The applied standard test procedures enable comparison between different vehicle types and different manufacturers.

3 Maximum performance determined by test procedure UN Global Technical Regulation-21 (Jan 2021).

4 Represents the lightest kerbweight assuming driver at 75 kg, full fluid levels and 90 per cent fuel levels, subject to manufacturing tolerances and options, etc., fitted. Weights represent base model specification without panoramic glass roof.

5 Towing limits quoted represent the maximum towing ability of the vehicle at its Gross Vehicle Mass to restart on a 12 per cent gradient at sea level. The performance and economy of all models will be reduced when used for towing.  Gross Train Mass includes trailer weight. Max towing varies based on cargo, vehicle configuration, accessories and number of passengers.

6 Measured in accordance with ISO 3832. Dimensions may vary dependent on the model and equipment fitted.

6 Cargo and load capacity limited by weight and weight distribution.

8 Driver-assist features are supplemental and do not replace the driver’s attention, judgment and need to control the vehicle. It does not replace safe driving. See Owner’s Manual for details and limitations.

9 Feature availability dependent on vehicle specification.

10 Cameras only operate at speeds under 10 km/h.

11 If stop is longer than three seconds, driver must intervene and press “RES” button or accelerator pedal to resume system operation.

12 Intersection Assist does not detect all vehicles in intersections, pedestrians, or cyclists. It does not replace safe driving. See Owner’s Manual for details and limitations.

13 Services require SYNC 4 or newer, activation through FordPass App (subject to FordPass Terms), modem activation, and a Connected Navigation service plan. Equipped vehicles come with a 1-year Connected Navigation included period from new vehicle warranty start date, after which purchase is required. Connected service and features depend on compatible network availability. Evolving technology/cellular networks/vehicle capability may limit functionality and prevent operation of connected features.

14 Requires phone with active data service and compatible software. SYNC 4 does not control 3rd party products while in use. 3rd Parties are solely responsible for their respective functionality.

15 To use Alexa Built-in, drivers need to activate FordPass within their vehicle using the FordPass smartphone app or in-vehicle settings. Access to Alexa requires an Amazon account, SYNC 4 or SYNC 4A, FordPass, and complimentary Connected Service activated through the FordPass app (see FordPass Terms for details). Additional products and setup may be required. Amazon, Alexa and all related logos and marks are trademarks of Amazon.com, Inc. or its affiliates.

16 Don’t drive while distracted or while using handheld devices. Use voice-operated systems when possible. Some features may be locked out while the vehicle is in gear. Not all features are compatible with all phones.

17 Available Qi wireless charging may not be compatible with all mobile phones.

Note: The data information in this press release was correct at the time of going to print. However, Ford policy is one of continuous product improvement. The right is reserved to change these details at any time.

Ford, a global American brand woven into the fabric of Europe for more than 100 years, is committed to freedom of movement that goes hand-in-hand with looking after the planet and each other. The company’s Ford+ plan, with Model e, Ford Pro and the Ford Blue business units is accelerating its European transformation to an all-electric and carbon neutral future by 2035. The company is driving forward with bold, new EVs, each one designed with European drivers in mind and innovating with services to help people connect, communities grow, and businesses thrive. Selling and servicing Ford vehicles in 50 individual European markets, operations also include the Ford Motor Credit Company, Ford Customer Service Division and 14 manufacturing facilities (eight wholly owned and six unconsolidated joint venture facilities) with four centres based in Cologne, Germany; Valencia, Spain and at our joint venture in Craiova, Romania and Kocaeli, Türkiye.  Ford employs approximately 34,000 people at its wholly owned facilities and consolidated joint ventures and approximately 57,000 people including unconsolidated businesses across Europe. More information about the company, its products and Ford Credit is available at corporate.ford.com.