All-NEW Ford FOCUS ACTIVE Specifications

Performance and Economy

| Focus Active 5-door |  |  |  | Fuel consumption l/100 km (mpg) |  |  | Performance |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Petrol engines | Power PS | Wheel size | CO2 g/km | Urban | Extra Urban | Combined | Max speed km/h (mph) | 0-100 km/h (0-62 mph) sec | 50-100 km/h (31-62 mph) sec* |
| 1.0 EcoBoost 6-speed manual | 125 | 17-inch | 107 | 5.9 (47.8) | 4.1 (68.9) | 4.8 (58.8) | 196 (122) | 10.3 | 10.4 |
|  |  | 18-inch | 110 | 6.0 (47.1) | 4.3 (65.7) | 4.9 (57.6) |  |  |  |
| 1.0 EcoBoost 8-speed automatic | 125 | 17-inch | 126 | 7.1 (39.8) | 4.7 (60.1) | 5.6 (50.4) | 191 (119) | 11.7 | N/A |
|  |  | 18-inch | 128 | 7.2 (39.2) | 4.8 (58.8) | 5.7 (49.5) |  |  |  |
| 1.5 EcoBoost 6-speed manual | 150 | 17-inch | 121 | 6.9 (40.9) | 4.5 (62.8) | 5.3 (53.3) | 206 (128) | 9.1 | 9.0 |
|  |  | 18-inch | 123 | 6.9 (40.9) | 4.6 (61.4) | 5.4 (52.3) |  |  |  |
| 1.5 EcoBoost 8-speed automatic | 150 | 17-inch | 133 | 7.6 (37.2) | 4.9 (57.6) | 5.9 (47.8) | 204 (127) | 9.7 | N/A |
|  |  | 18-inch | 136 | 7.7 (36.7) | 5.0 (56.5) | 6.0 (47.1) |  |  |  |
| 1.5 EcoBoost 8-speed automatic | 182 | 17-inch | 132 | 7.5 (37.7) | 4.9 (57.6) | 5.9 (47.8) | 216 (134) | 8.7 | N/A |
|  |  | 18-inch | 135 | 7.6 (37.2) | 5.0 (56.5) | 6.0 (47.1) |  |  |  |

| Focus Active 5-door |  |  |  | Fuel consumption l/100 km (mpg) |  |  | Performance |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Diesel engines | Power PS | Wheel size | CO2 g/km | Urban | Extra Urban | Combined | Max speed km/h (mph) | 0-100 km/h (0-62 mph) sec | 50-100 km/h (31-62 mph) sec* |
| 1.5 EcoBlue 6-speed manual | 120 | 17-inch | 93 | 3.9 (72.4) | 3.3 (85.6) | 3.5 (80.7) | 192 (119) | 10.8 | 11.0 |
|  |  | 18-inch | 96 | 4.0 (70.6) | 3.4 (83.1) | 3.6 (78.5) |  |  |  |
| 1.5 EcoBlue 8-speed automatic | 120 | 17-inch | 110 | 4.7 (60.1) | 3.9 (72.4) | 4.2 (67.2) | 189 (117) | 10.7 | N/A |
|  |  | 18-inch | 112 | 4.8 (58.8) | 4.0 (70.6) | 4.3 (65.7) |  |  |  |
| 2.0 EcoBlue 6-speed manual | 150 | 17-inch | 114 | 5.6 (50.4) | 3.6 (78.5) | 4.4 (64.2) | 206 (128) | 9.1 | 8.7 |
|  |  | 18-inch | 115 | 5.6 (50.4) | 3.6 (78.5) | 4.4 (64.2) |  |  |  |
| 2.0 EcoBlue 8-speed automatic | 150 | 17-inch | 114 | 5.3 (53.3) | 3.9 (72.4) | 4.4 (64.2) | 205 (127) | 9.3 | N/A |
|  |  | 18-inch | 117 | 5.5 (51.4) | 4.0 (70.6) | 4.5 (62.8) |  |  |  |

| Focus Active wagon |  |  |  | Fuel consumption l/100 km (mpg) |  |  | Performance |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Petrol engines | Power PS | Wheel size | CO2 g/km | Urban | Extra Urban | Combined | Max speed km/h (mph) | 0-100 km/h (0-62 mph) sec | 50-100 km/h (31-62 mph) sec* |
| 1.0 EcoBoost 6-speed manual | 125 | 17-inch | 108 | 5.9 (47.8) | 4.1 (68.9) | 4.8 (58.8) | 194 (120) | 10.6 | 10.8 |
|  |  | 18-inch | 111 | 6.0 (47.1) | 4.3 (65.7) | 4.9 (57.6) |  |  |  |
| 1.0 EcoBoost 8-speed automatic | 125 | 17-inch | 130 | 7.3 (38.7) | 4.9 (57.6) | 5.8 (48.7) | 189 (117) | 12.0 | N/A |
|  |  | 18-inch | 132 | 7.4 (38.2) | 5.0 (56.5) | 5.9 (47.8) |  |  |  |
| 1.5 EcoBoost 6-speed manual | 150 | 17-inch | 125 | 7.0 (40.3) | 4.7 (60.1) | 5.5 (51.4) | 204 (127) | 9.3 | 9.3 |
|  |  | 18-inch | 127 | 7.1 (39.8) | 4.7 (60.1) | 5.6 (50.4) |  |  |  |
| 1.5 EcoBoost 8-speed automatic | 150 | 17-inch | 134 | 7.6 (37.2) | 5.0 (56.5) | 5.9 (47.8) | 202 (125) | 9.9 | N/A |
|  |  | 18-inch | 136 | 7.7 (36.7) | 5.0 (56.5) | 6.0 (47.1) |  |  |  |
| 1.5 EcoBoost 8-speed automatic | 182 | 17-inch | 133 | 7.5 (37.7) | 4.9 (57.6) | 5.9 (47.8) | 214 (133) | 8.9 | N/A |
|  |  | 18-inch | 135 | 7.6 (37.2) | 5.0 (56.5) | 6.0 (47.1) |  |  |  |

| Focus Active wagon |  |  |  | Fuel consumption l/100 km (mpg) |  |  | Performance |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Diesel engines | Power PS | Wheel size | CO2 g/km | Urban | Extra Urban | Combined | Max speed km/h (mph) | 0-100 km/h (0-62 mph) sec | 50-100 km/h (31-62 mph) sec* |
| 1.5 EcoBlue 6-speed manual | 120 | 17-inch | 97 | 4.1 (68.9) | 3.5 (80.7) | 3.7 (76.3) | 190 (118) | 11.1 | 11.3 |
|  |  | 18-inch | 100 | 4.2 (67.2) | 3.6 (78.5) | 3.8 (74.3) |  |  |  |
| 1.5 EcoBlue 8-speed automatic | 120 | 17-inch | 116 | 4.9 (57.6) | 4.1 (68.9) | 4.4 (64.2) | 187 (116) | 11.0 | N/A |
|  |  | 18-inch | 119 | 5.1 (55.4) | 4.2 (67.2) | 4.5 (62.8) |  |  |  |
| 2.0 EcoBlue 6-speed manual | 150 | 17-inch | 117 | 5.7 (49.5) | 3.8 (74.3) | 4.5 (62.8) | 204 (127) | 9.3 | 9.0 |
|  |  | 18-inch | 118 | 5.7 (49.5) | 3.8 (74.3) | 4.6 (61.4) |  |  |  |
| 2.0 EcoBlue 8-speed automatic | 150 | 17-inch | 116 | 5.3 (53.3) | 3.9 (72.4) | 4.4 (64.2) | 203 (126) | 9.5 | N/A |
|  |  | 18-inch | 119 | 5.5 (51.4) | 4.1 (68.9) | 4.6 (61.4) |  |  |  |

* In 4th gear

WEIGHTS AND DIMENSIONS

Weights

| Focus Active 5-door |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Kerb weight (kg)# | Gross Vehicle Mass (kg) | Gross Train Mass (kg) | Max. Towable Mass (braked) (kg) | Max. Towable Mass (unbraked) (kg) | Nose weight (kg) | Roof load (kg) |
| 1.0 EcoBoost 125 PS 6-speed manual | 1322 | 1855 | 2955 | 1100 | 660 | 90 | 75 |
| 1.0 EcoBoost 125 PS 8-speed automatic | 1371 | 1905 | 3205 | 1300 | 685 | 90 | 75 |
| 1.5 EcoBoost 150 PS 6-speed manual | 1369 | 1890 | 3390 | 1500 | 680 | 90 | 75 |
| 1.5 EcoBoost 150 PS 8-speed automatic | 1404 | 1905 | 3405 | 1500 | 700 | 75 | 75 |
| 1.5 EcoBoost 182 PS 8-speed automatic | 1404 | 1905 | 3405 | 1500 | 700 | 75 | 75 |
| 1.5 EcoBlue 120 PS 6-speed manual | 1363 | 1895 | 3295 | 1400 | 680 | 90 | 75 |
| 1.5 EcoBlue 120 PS 8-speed automatic | 1394 | 1910 | 3310 | 1400 | 695 | 80 | 75 |
| 2.0 EcoBlue 150 PS 6-speed manual | 1493 | 2020 | 3820 | 1800 | 745 | 90 | 75 |
| 2.0 EcoBlue 150 PS 8-speed automatic | 1518 | 2045 | 3645 | 1600 | 750 | 90 | 75 |

| Focus Active wagon |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Kerb weight (kg)# | Gross Vehicle Mass (kg) | Gross Train Mass (kg) | Max. Towable Mass (braked) (kg) | Max. Towable Mass (unbraked) (kg) | Nose weight (kg) | Roof load (kg) |
| 1.0 EcoBoost 125 PS 6-speed manual | 1383 | 1925 | 2925 | 1000 | 690 | 90 | 75 |
| 1.0 EcoBoost 125 PS 8-speed automatic | 1431 | 1970 | 3270 | 1300 | 715 | 90 | 75 |
| 1.5 EcoBoost 150 PS 6-speed manual | 1408 | 1950 | 3450 | 1500 | 700 | 90 | 75 |
| 1.5 EcoBoost 150 PS 8-speed automatic | 1445 | 1980 | 3480 | 1500 | 720 | 90 | 75 |
| 1.5 EcoBoost 182 PS 8-speed automatic | 1445 | 1980 | 3480 | 1500 | 720 | 90 | 75 |
| 1.5 EcoBlue 120 PS 6-speed manual | 1413 | 1955 | 3355 | 1400 | 705 | 80 | 75 |
| 1.5 EcoBlue 120 PS 8-speed automatic | 1454 | 1995 | 3395 | 1400 | 725 | 90 | 75 |
| 2.0 EcoBlue 150 PS 6-speed manual | 1553 | 2080 | 3880 | 1800 | 750 | 90 | 75 |
| 2.0 EcoBlue 150 PS 8-speed automatic | 1559 | 2105 | 3705 | 1600 | 750 | 90 | 75 |

# Represents the lightest kerbweight assuming driver at 75 kg, full fluid levels and 90 per cent fuel levels, subject to manufacturing tolerances and options, etc., fitted. Weights represent base model specification.

Towing limits quoted represent the maximum towing ability of the vehicle at its Gross Vehicle Mass to restart on a 12 per cent gradient at sea level. The performance and economy of all models will be reduced when used for towing.  Gross Train Mass includes trailer weight.

Dimensions

| Dimensions (mm) | Focus 5-door | Focus Wagon |
| --- | --- | --- |
| Exterior |  |  |
| Overall length | 4397 | 4693 |
| Overall width with/without/folded mirrors | 1979/1825/1848 | 1979/1825/1848 |
| Overall height (including antenna) | 1502 | 1532 |
| Wheelbase | 2700 | 2700 |
| Front track | 1582 | 1582 |
| Rear track | 1576 | 1576 |
| Front overhang | 918 | 918 |
| Rear overhang | 779 | 1075 |
| Min ground clearance (unloaded) | 163 | 163 |
| Min ground clearance (loaded) | 141 | 149 |
| Interior |  |  |
| Front headroom | 987 | 987 |
| Front max legroom | 1109 | 1109 |
| Front shoulder room | 1421 | 1421 |
| Front hip room | 1364 | 1364 |
| Rear headroom | 965 | 1008 |
| Rear legroom | 918 | 918 |
| Rear shoulder room | 1395 | 1395 |
| Rear hip room | 1325 | 1325 |
| Luggage capacity (litres) ‡ |  |  |
| 5-seat mode, laden to package tray (with tyre repair kit) | 375 | 608 |
| 2-seat mode, laden to roof (with tyre repair kit) | 1354 | 1653 |
| Luggage compartment dimensions (mm) |  |  |
| Load length at floor to 1st row (tyre repair kit) | 1624 | 1835 |
| Load length at floor to 2nd row (tyre repair kit) | 844 | 1047 |
| Load width between wheelhouses | 1016 | 1150 |
| Load opening width (max) | 1018 | 1051 |
| Load opening width (floor) | 998 | 1078 |
| Load opening height | 613 | 702 |
| Lift-over height (kerb) | 712 | 665 |
| Fuel tank capacity (litres) |  |  |
| Petrol/Diesel | 52/47 | 52/47 |

‡ Measured in accordance with ISO 3832. Dimensions may vary dependent on the model and equipment fitted. Dimensions represent base model specification.

DRIVER ASSISTANCE TECHNOLOGIES

Active Park Assist 2 with Park-Out Assist

Adaptive Cruise Control with Stop & Go, Speed Sign Recognition and Lane-Centring

Adaptive Front Lighting System with Predictive curve light and Sign-based light

Adjustable Speed Limiter

Auto High Beam

Blind Spot Information System

Cross Traffic Alert with Active Braking

Distance Indication

Drive Modes – Normal, Sport, Eco, Slippery, Trail

Driver Alert

Evasive Steering Assist

Forward Collision Warning

Front and Rear Park Aid

Glare-free high beam

Head-up display

Hill Launch Assist

Intelligent Speed Limiter

Lane Keeping Aid

Lane Keeping Alert

Post-Collision Braking

Pre-Collision Assist with Pedestrian and Cyclist Detection

Rear wide-view camera

Traffic Sign Recognition

Wrong Way Alert

COMFORT AND CONVENIENCE

B&O PLAY Sound System

Easy Fold Seats

Electric parking brake

FordPass Connect embedded modem

Hands-free tailgate

Heated steering wheel

Openable panorama roof

Rotary gear shift dial

SYNC 3 with 8-inch touchscreen

Wireless charging pad

STEERING

| System | Rack and pinion with column-mounted electronic-power assisted steering module |
| --- | --- |
| Ratio | 14.3:1 |
| Turning circle (m) kerb-to-kerb | 11.0 (17-inch tyres) 11.6 (18-inch tyres) |

CHASSIS

| Front suspension | Independent MacPherson struts with offset coil spring over gas filled twin-tube damper units and lower L-arms with optimised front rubber bushings and rear rubber bushings (or hydro-bush on selected powertrains) mounted on separate reinforced cross-member sub-frame, anti-roll bar. Unique Focus Active steering knuckle geometry, spring and damper tuning. |
| --- | --- |
| Rear suspension | Short-long-arm, multi-link independent geometry with isolated subframe. Unique geometry for wagon models. Twin-tube rear damper with optimised dual path body mounts. Unique dampers for wagon models. Unique Focus Active steering knuckle geometry, spring and damper tuning. |

BRAKES

|  | Front | Rear |
| --- | --- | --- |
| Braking | Hydraulically operated dual-circuit system with diagonal distribution. Vented front discs. Solid rear discs. Electronic four-channel anti-lock braking system (ABS) with electronic brake-force distribution (EBD), Electronic Stability Program (ESP) and Emergency Brake Assist (EBA). Autonomous emergency braking (AEB) as part of Pre-Collision Assist with Pedestrian Detection. Electric Brake Booster for selected models |  |
| Disc dimensions (mm) | Ø282 x 27 or Ø308 x 27 depending on vehicle configuration | Ø271 x 12 or Ø302 x 12 depending on vehicle configuration |
| Piston dimensions (mm) | Ø57 | Ø36 or Ø38 depending on vehicle configuration |

WHEELS & TYRES

17-inch with 215/55 R17

18-inch with 215/50 R18

PETROL ENGINES

|  |  | 1.0-litre EcoBoost (125 PS) |  |
| --- | --- | --- | --- |
| Type |  | In-line three cylinder turbo petrol, Ti-VCT, transverse |  |
| Displacement | cm3 | 999 |  |
| Bore | Mm | 71.9 |  |
| Stroke | Mm | 82.0 |  |
| Compression ratio |  | 10.5:1 |  |
| Max power | PS (kW) | 125 (92) |  |
|  | at rpm | 6000 |  |
| Max torque | Nm | 170 (200 overboost) |  |
|  | at rpm | 1400-4500 |  |
| Valve gear |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |  |
| Cylinders |  | 3 in-line, cylinder deactivation technology |  |
| Cylinder head |  | Cast aluminium |  |
| Cylinder block |  | Cast iron |  |
| Camshaft drive |  | Chain drive with hydraulic tensioner |  |
| Crankshaft |  | Cast iron, 6 counterweights, 4 main bearings |  |
| Engine management |  | Bosch MED17 with CAN-Bus and individual cylinder knock control. FGEC software |  |
| Fuel injection |  | High pressure direct fuel injection with 5 hole injectors |  |
| Emission level |  | Euro 6d-TEMP |  |
| Emission control |  | Rapid light-off catalyst, gasoline particulate filter |  |
| Turbocharger |  | Continental radial-axial (RAAX) turbocharger |  |
| Lubrication system |  | Electronically controlled variable displacement oil pump for improved fuel economy |  |
| System capacity with filter | Litres | 4.6 |  |
| Cooling system |  | Split cooling system with 2 thermostats. Cam driven water pump for reduced power consumption |  |
| System capacity incl heater | Litres | 6.0 manual/6.4 auto |  |
| Transmission |  | 6-speed manual | 8-speed automatic |
| Gear ratios |  | 6th 0.634 5th 0.757 4th 0.943 3rd 1.276 2nd 1.958 1st 3.417 R 3.895 FDR 4.353 | 8th 0.694 7th 0.864 6th 1.000 5th 1.420 4th 2.036 3rd 2.645 2nd 3.417 1st 5.324 Reverse 4.293 FDR: 3.52 |

|  |  | 1.5-litre EcoBoost (150, 182 PS) |  |  |
| --- | --- | --- | --- | --- |
| Type |  | In-line three cylinder turbo petrol, Ti-VCT, transverse |  |  |
| Displacement | cm3 | 1497 |  |  |
| Bore | Mm | 84.0 |  |  |
| Stroke | Mm | 90.0 |  |  |
| Compression ratio |  | 11.0:1 |  |  |
| Max power | PS (kW) | 150 (110) |  | 182 (134) |
|  | at rpm | 6000 |  | 6000 |
| Max torque | Nm | 240 |  | 240 |
|  | at rpm | 1600 |  | 1600 |
| Valve gear |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |  |  |
| Cylinders |  | 3 in-line, cylinder deactivation technology |  |  |
| Cylinder head |  | Cast aluminium |  |  |
| Cylinder block |  | Cast aluminium |  |  |
| Camshaft drive |  | Chain driven with hydraulic tensioner |  |  |
| Crankshaft |  | Cast iron, 6 counterweights, 4 main bearings |  |  |
| Engine management |  | Bosch MG1CS016 with CAN-Bus and individual cylinder knock control. FGEC software |  |  |
| Fuel injection |  | 3 high-pressure direct fuel injection, 6 hole injectors (DI) plus 3 low-pressure port injectors (PFI) |  |  |
| Emission level |  | Euro 6d-TEMP |  |  |
| Emission control |  | Rapid light-off catalyst, gasoline particulate filter |  |  |
| Turbocharger |  | Continental radial-axial (RAAX) turbocharger |  |  |
| Lubrication system |  | Two-stage variable displacement oil pump |  |  |
| System capacity with filter | Litres | 5.25 |  |  |
| Cooling system |  | Single thermostat |  |  |
| System capacity incl heater | Litres | 6.1 manual/6.5 auto |  |  |
| Transmission |  | 6-speed manual | 8-speed automatic | 8-speed automatic |
| Gear ratios |  | 6th 0.651 5th 0.775 4th 0.971 3rd 1.290 2nd 1.952 1st 3.583 Reverse 3.333 FDR1: 4.056 FDR2: 4.563 | 8th 0.694 7th 0.864 6th 1.000 5th 1.420 4th 2.036 3rd 2.645 2nd 3.417 1st 5.324 Reverse 4.293 FDR: 2.940 | 8th 0.694 7th 0.864 6th 1.000 5th 1.420 4th 2.036 3rd 2.645 2nd 3.417 1st 5.324 Reverse 4.293 FDR: 2.940 |

DIESEL ENGINES

|  |  | 1.5-litre EcoBlue (120 PS) |  |
| --- | --- | --- | --- |
| Type |  | In-line four cylinder turbo diesel, transverse |  |
| Displacement | cm3 | 1500 |  |
| Bore | Mm | 75.0 |  |
| Stroke | Mm | 84.8 |  |
| Compression ratio |  | 16.4:1 |  |
| Max power | PS (kW) | 120 (88) 3600 300 1750-2250 |  |
|  | at rpm |  |  |
| Max torque | Nm |  |  |
|  | at rpm |  |  |
| Valve gear |  | DOHC with 4 valves per cylinder |  |
| Cylinders |  | 4 in-line |  |
| Cylinder head |  | Cast aluminium |  |
| Cylinder block |  | Cast aluminium |  |
| Camshaft drive |  | Low-friction, Kevlar-reinforced primary drive belt. Cam-to-cam chain |  |
| Crankshaft |  | Steel forging, 4 counter-weights, 5 main bearings |  |
| Engine management |  | Bosch FDEC |  |
| Fuel injection |  | Common rail direct fuel injection; 2000 bar injection pressure; 8-hole injectors |  |
| Emission level |  | Euro 6d-TEMP |  |
| Emission control |  | High- and low-pressure exhaust gas recirculation; water-air charge cooling; Twin lean NOx trap |  |
| Turbocharger |  | Low-inertia variable geometry turbocharger with electronic control |  |
| Lubrication system |  | Pressure-fed lubrication system with full flow oil filter, variable displacement oil pump |  |
| System capacity with filter | litres | 3.8 |  |
| Cooling system |  | Water pump with electrically switchable thermostat |  |
| System capacity | litres | 6.7 manual/7.3 auto |  |
| Transmission |  | 6-speed manual | 8-speed automatic |
| Gear ratios |  | 6th 0.578 5th 0.690 4th 0.868 3rd 1.194 2nd 1.952 1st 3.727 Reverse 3.455 FDR1: 3.526 FDR2: 3.941 | 8th 0.62 7th 0.75 6th 1.00 5th 1.45 4th 1.92 3rd 3.01 2nd 3.31 1st 4.69 Reverse 2.96 FDR: 3.17 |

|  |  | 2.0-litre EcoBlue (150 PS) |
| --- | --- | --- |
| Type |  | Inline four cylinder turbo diesel, transverse |
| Displacement | cm3 | 1996 |
| Bore | Mm | 84.0 |
| Stroke | Mm | 90.0 |
| Compression ratio |  | 15.8:1 |
| Max power | PS (kW) | 150 (110) |
|  | at rpm | 3500 |
| Max torque | Nm | 370 |
|  | at rpm | 2000-2500 |
| Valve gear |  | DOHC with 4 valves per cylinder |
| Cylinders |  | 4 inline |
| Cylinder head |  | Cast aluminium |
| Cylinder block |  | Cast iron |
| Camshaft drive |  | Low-friction belt-in-oil with dynamic tensioner |
| Crankshaft |  | Forged steel, 4 counter-weights, 5 main bearings |
| Engine management |  | Ford Common Rail Diesel Engine Management System |
| Fuel injection |  | Common rail direct fuel injection; 2200 bar injection pressure; 8-hole piezo-electric injectors |
| Emission level |  | Euro 6d-TEMP |
| Emission control |  | High- and low-pressure exhaust gas recirculation; water-air charge cooling; selective catalytic reduction after treatment |
| Turbocharger |  | Low-inertia variable geometry turbocharger with electronic control |
| Lubrication system |  | Pressure-fed lubrication system with full flow oil filter, variable displacement oil pump |
| System capacity with filter | litres | 7.25 |
| Cooling system |  | Engine driven water pump with thermostat control |
| System capacity | litres | 8.3 |
| Transmission |  | 8-speed automatic |
| Gear ratios |  | 8th 0.62 7th 0.75 6th 1.00 5th 1.45 4th 1.92 3rd 3.01 2nd 3.31 1st 4.69 Reverse 2.96 FDR: 2.89 |

# # #

Note: The data information in this press release reflects preliminary specifications and was correct at the time of going to print. However, Ford policy is one of continuous product improvement. The right is reserved to change these details at any time.

Note: The declared Fuel/Energy Consumptions, CO2 emissions and electric range are measured according to the technical requirements and specifications of the European Regulations (EC) 715/2007 and (EC) 692/2008 as last amended. Fuel consumption and CO2 emissions are specified for a vehicle variant and not for a single car. The applied standard test procedure enables comparison between different vehicle types and different manufacturers. In addition to the fuel-efficiency of a car, driving behaviour as well as other non-technical factors play a role in determining a car's fuel/energy consumption, CO2 emissions and electric range. CO2 is the main greenhouse gas responsible for global warming.

From 1 September 2017, certain new vehicles will be type-approved using the World Harmonised Light Vehicle Test Procedure (WLTP) according (EU) 2017/1151 as last amended, which is a new, more realistic test procedure for measuring fuel consumption and CO2 emissions. From 1 September 2018 the WLTP will fully replace the New European Drive Cycle (NEDC), which is the current test procedure. During NEDC Phase-out, WLTP fuel consumption and CO2 emissions are being correlated back to NEDC. There will be some variance to the previous fuel economy and emissions as some elements of the tests have altered i.e., the same car might have different fuel consumption and CO2 emissions.

About Ford Motor Company

Ford Motor Company is a global company based in Dearborn, Michigan. The company designs, manufactures, markets and services a full line of Ford cars, trucks, SUVs, electrified vehicles and Lincoln luxury vehicles, provides financial services through Ford Motor Credit Company and is pursuing leadership positions in electrification, autonomous vehicles and mobility solutions. Ford employs approximately 200,000 people worldwide. For more information regarding Ford, its products and Ford Motor Credit Company, please visit www.corporate.ford.com.

Ford of Europe is responsible for producing, selling and servicing Ford brand vehicles in 50 individual markets and employs approximately 53,000 employees at its wholly owned facilities and approximately 68,000 people when joint ventures and unconsolidated businesses are included. In addition to Ford Motor Credit Company, Ford Europe operations include Ford Customer Service Division and 24 manufacturing facilities (16 wholly owned or consolidated joint venture facilities and eight unconsolidated joint venture facilities). The first Ford cars were shipped to Europe in 1903 – the same year Ford Motor Company was founded. European production started in 1911.

|  | Contact:; Finn Thomasen. Ford of Europe. +44 (0) 1268 401 908. fthomas3@ford.com.. |
| --- | --- |