Ford EXPLORER PLUG-IN HYBRID Specifications

Performance and Economy

| Hybrid engines |  |  | CO2 from (g/km NEDC) | Fuel consumption from (l/100 km NEDC) |  | Fuel consumption from (l/100 km WLTP) | CO2 from (g/km WLTP) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Power PS | Wheel size inch | Combined | Combined |  | Overall | Overall |
| 3.0-litre EcoBoost Plug-In Hybrid 10-speed automatic | 457 | 20 | 66 | 2.9 |  | 3.1 | 71 |
|  |  | Electric range NEDC |  |  | 48 km |  |  |
|  |  | Equivalent all-electric range overall WLTP |  |  | 42 km |  |  |
|  |  | Equivalent all-electric range city WLTP |  |  | 44 km |  |  |

|  |  | Performance |  |
| --- | --- | --- | --- |
| Hybrid engines | Power PS | Max speed km/h | 0-100 km/h sec |
| 3.0-litre EcoBoost Plug-In Hybrid 10-speed automatic | 457 | 230 | 6.0 |

WEIGHTS AND DIMENSIONS

|  | Kerb weight (kg)# | Gross Vehicle Mass (kg) | Gross Train Mass (kg) | Max. Towable Mass (braked) (kg) | Max. Towable Mass (unbraked) (kg) | Nose weight (kg) | Roof load (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.0-litre EcoBoost Plug-In Hybrid 10-speed automatic | 2466 | 3160 | 5660 | 2500 | 750 | 100 | 35 |

# Represents the lightest kerbweight assuming driver at 75 kg, full fluid levels and 90 per cent fuel levels, subject to manufacturing tolerances and options, etc., fitted.

Towing limits quoted represent the maximum towing ability of the vehicle at its Gross Vehicle Mass to restart on a 12 per cent gradient at sea level. The performance and economy of all models will be reduced when used for towing.  Gross Train Mass includes trailer weight.

Dimensions

| Dimensions (mm unless stated) | ST-Line/Platinum |
| --- | --- |
| Exterior |  |
| Overall length without tow bar | 5063 |
| Overall width mirrors extended | 2285 |
| Overall width mirrors folded | 2107 |
| Overall height with roof rails | 1783 |
| Wheelbase | 3025 |
| Front track | 1704 |
| Rear track | 1713 |
| Front overhang | 869/855 |
| Rear overhang | 1169 |
| Min ground clearance (GVM) | 204 |
| Min ground clearance (kerb) | 167 |
| Approach Angle (degrees) | 20.2 |
| Departure Angle (degrees) | 21.0 |
| Interior |  |
| First-row headroom | 987 |
| First-row legroom | 1093 |
| First-row shoulder room | 1570 |
| First-row hip room | 1504 |
| Second-row headroom | 971 |
| Second-row legroom | 991 |
| Second-row shoulder room | 1573 |
| Second-row hip room | 1501 |
| Third-row headroom | 988 |
| Third-row legroom | 817 |
| Third-row shoulder room | 1388 |
| Third-row hip room | 1038 |
| Luggage capacity (litres) ‡ |  |
| 7-seat mode, laden to package tray (with temporary spare) | 240 |
| 7-seat mode, laden to roof (with temporary spare) | 330 |
| 5-seat mode, laden to package tray (with temporary spare) | 635 |
| 5-seat mode, laden to roof (with temporary spare) | 1137 |
| 2-seat mode, laden to roof (with temporary spare) | 2274 |
| Luggage compartment dimensions |  |
| Maximum loading height | 855 |
| Load length at floor to 1st row seats | 2132 |
| Load length at floor to 2nd row seats | 1264 |
| Load length at floor to 3rd row seats | 532 |
| Load width between wheelhouses | 1222 |
| Load opening width (floor) | 1216 |
| Lift-over height (kerb) | 804 |
| Fuel tank capacity (litres) |  |
| Petrol | 68.4 |

‡ Measured in accordance with ISO 3832. Dimensions may vary dependent on the model and equipment fitted.

DRIVER ASSISTANCE TECHNOLOGIES≠

360-Degree Camera including front and rear wide-view cameras

Active Park Assist 2

Adaptive Cruise Control with Stop & Go, Speed Sign Recognition and Lane Centring

Auto High Beam

Auto Hold

Auto lighting and rain-sensing wipers

Blind Spot Information System with Cross-Traffic Alert

EcoCoach

Evasive Steering Assist

Forward Collision Warning

Front and rear parking sensors

Hill Start Assist

Hill Decent Assist

Intelligent Speed Limiter

Intelligent All-Wheel Drive

Lane-Keeping System

Post-Collision Braking

Pre-Collision Assist with Active Braking

Reverse Brake Assist

Side-Wind Stabilisation

Terrain Management System

Traffic Sign Recognition

Trailer Sway Control

COMFORT AND CONVENIENCE≠

10-way Power adjustable driver and passenger seat with memory on driver’s seat

12.3-inch digital instrument cluster

Active Noise Cancellation

Acoustic laminated glass

Ambient Lighting

B&O Sound System

Dual first-row smart-charging multimedia USB ports

Dual second-row smart-charging USB ports

Electrically-adjustable steering column

Five 12-volt power points

FordPass Connect modem

Hands-free tailgate

Heated leather steering wheel

Heated and ventilated front seats

Heated second-row seats

Multicontour seats with Active Motion

Openable panoramic roof

Paddle shifters

SYNC 3 with 10.1-inch capacitive touchscreen

Third row Easy Fold Seats with Power Raise

Tri-Zone Electronic Temperature Control with cabin particulate filter

Windscreen wiper de-icer

Wireless charging pad

≠Feature availability dependent on vehicle specification

STEERING

| System | Rack and pinion with Electronic Power Assisted Steering (EPAS) |
| --- | --- |
| Ratio | 16.5:1 |
| Turning circle (m) kerb-to-kerb | 12.6 |

CHASSIS

| Front suspension | Independent. MacPherson struts with 32 mm stabiliser bar, gas-pressurised hydraulic shock absorbers |
| --- | --- |
| Rear suspension | Independent multi-link rear suspension with 22 mm stabiliser bar, gas-pressurised hydraulic shock absorbers |

BRAKES

|  | Front | Rear |
| --- | --- | --- |
| Braking | Hydraulically operated dual-circuit system with diagonal distribution. Vented front discs, solid rear discs. Electronic four-channel anti-lock braking system (ABS) with electronic brake-force distribution (EBD), Electronic Stability System (ESP) and Emergency Brake Assist (EBA), Autonomous emergency braking (AEB) as part of Pre-Collision Assist with Active Braking. |  |
| Disc/Drum dimensions (mm) | Ø363 x 32 | Ø350 x 26 disc |
| Piston dimensions (mm) | Ø48 | Ø45 |

WHEELS & TYRES

8 x 20-inch with 255/55 R20

8.5 x 20-inch with 255/55 R20

HYBRID ENGINES

|  |  | 3.0-litre EcoBoost Plug-In Hybrid (457 PS) |
| --- | --- | --- |
| Type |  | 3.0-litre V6 turbocharged petrol |
| Displacement | cm3 | 3000 |
| Bore | mm | 85.4 |
| Stroke | mm | 86.0 |
| Compression ratio |  | 9.5:1 |
| Motor |  | 75 kW |
| Battery |  | Lithium-ion |
| Battery capacity |  | 13.6 kWh, 36 Ah |
| Max power (combined) | PS (kW) | 457 (336) |
|  | at rpm | 5750 |
| Max torque steady state | Nm | 825 |
|  | at rpm | 2500 |
| Valve gear Cylinders |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |
|  |  | 60-degree V6 |
| Cylinder head |  | Cast aluminium |
| Cylinder block |  | Compacted graphite iron aluminium |
| Camshaft drive |  | Chain drive |
| Crankshaft |  | Forged steel, 6 counterweights, 4 main bearings |
| Engine management |  | Bosch MG1CS018 with CAN-Bus and individual cylinder knock control. FGEC software |
| Fuel injection |  | High pressure direct fuel injection |
| Emission level |  | Euro 6d-TEMP |
| Emission control |  | Rapid light-off catalyst, gasoline particulate filter |
| Turbocharger |  | Twin fixed geometry turbochargers |
| Lubrication system |  | Electronically controlled continuously variable displacement oil pump for improved fuel economy |
| Engine cooling system |  | Single thermostat, accessory-belt driven |
| Transmission |  | 10-speed automatic |
| Gear ratios |  | 10th 0.636 9th 0.689 8th 0.853 7th 1.000 6th 1.275 5th 1.521 4th 1.769 3rd 2.149 2nd 2.997 1st 4.714 R 4.885 FDR 3.31 |

# # #

Note: The data information in this press release reflects preliminary specifications and was correct at the time of going to print. However, Ford policy is one of continuous product improvement. The right is reserved to change these details at any time.

The declared fuel/energy consumptions, CO2-emissions and electric range are determined according to the technical requirements and specifications of the European Regulations (EC) 715/2007 and (EU) 2017/1151 as last amended. Light Duty Vehicle type-approved using the World Harmonised Light Vehicle Test Procedure (WLTP) will have fuel/energy consumption and CO2-emission information for New European Drive Cycle (NEDC) and WLTP. WLTP will fully replace the NEDC latest by the end of the year 2020. The applied standard test procedures enable comparison between different vehicle types and different manufacturers. During NEDC phase-out, WLTP fuel consumption and CO2 emissions are being correlated back to NEDC. There will be some variance to the previous fuel economy and emissions as some elements of the tests have altered, so the same car might have different fuel consumption and CO2 emissions.

About Ford Motor Company

Ford Motor Company is a global company based in Dearborn, Michigan. The company designs, manufactures, markets and services a full line of Ford cars, trucks, SUVs, electrified vehicles and Lincoln luxury vehicles, provides financial services through Ford Motor Credit Company and is pursuing leadership positions in electrification; mobility solutions, including self-driving services; and connected services.  Ford employs approximately 188,000 people worldwide. For more information regarding Ford, its products and Ford Motor Credit Company, please visit www.corporate.ford.com.

Ford of Europe is responsible for producing, selling and servicing Ford brand vehicles in 50 individual markets and employs approximately 45,000 employees at its wholly owned facilities and consolidated joint ventures and approximately 59,000 people when unconsolidated businesses are included. In addition to Ford Motor Credit Company, Ford Europe operations include Ford Customer Service Division and 19 manufacturing facilities (12 wholly owned facilities and seven unconsolidated joint venture facilities). The first Ford cars were shipped to Europe in 1903 – the same year Ford Motor Company was founded. European production started in 1911.

| Contact: | Finn Thomasen |  |  |
| --- | --- | --- | --- |
|  | Ford of Europe |  |  |
|  | +44 (0) 1268 401 908 fthomas3@ford.com |  |  |